MINI USB Nano V3.0 ATmega328P CH340G 5V 16M Micro-controller board Arduino

Specification:

CH340G Replace FT232RL

Operating Voltage:5v

8 analog inputs ports: A0~A7

14 digital input/output ports: TX, RX, D2~D13

6PWM ports: D3, D5,D6,D9,D10,D11

1 Pairs of TTL level serial transceiver ports RX/TX

Using atmel atmega328P-AU MCU

There is bootloader installed in it

Support usb download and power

Support for external 5V~12V DC power supply

Support power supply by 9V battery

Package list:

1 x mini usb Nano V3.0 ATmega328P CH340G 5V 16M Micro-controller board